Our route servers support the following communities to control who sees your prefixes:

Standard communities

0:{peer-as} - Do not advertise to {peer-as}

37195:{peer-as} - Advertise to {peer-as}

0:37195 - Advertise to nobody

37195:37195 - Advertise to everybody

Extended communities

(ro,666,{peer-as}) - Do not advertise to {peer-as}

(ro,37195,{peer-as}) - Advertise to {peer-as}

(ro,666,37195) - Advertise to nobody

(ro,37195,37195) - Advertise to everybody

Large communities

(37195, 0, {peer-as}) - Do not advertise to {peer-as}

(37195, 1, {peer-as}) - Advertise to {peer-as}

(37195, 0, 0) - Advertise to nobody

(37195, 1, 0) - Advertise to everybody

Path prepending

(37195, 101, {peer-as}) - Prepend once to {peer-as}

(37195, 102, {peer-as}) - Prepend twice to {peer-as}

(37195, 103, {peer-as}) - Prepend thrice to {peer-as}

You will need to send either "Advertise to everybody, except specifics" OR "Advertise to nobody, except specifics.